시 방 서

| 제품명 | WT3000 전력분석기 |
| --- | --- |
| 용도설명 | - 미소 전류 측정 및 대전류 측정 - 직·교류겸용 단상(3상)회로 측정 가능 - 전압, 전류, 유효전력, 피상전력, 무효전력, 역률등을 측정 가능 - 가전, 전기기기, 전원, 조명등 고주파를 포함한 파형, 교류 혼재파형의 고정밀 측정. - 전압 레인지 최소 15 V~ 최대 1000 V까지 대응. - 전압, 전류등의 수치 데이터 뿐만이 아니고, 파형, 바 그래프, 벡터, 측정 데이터의 Trend (시간당 파형기록) 등의 표시 기능이 탑재. - 기본 전력 확도 : ±0.02% reading, DC and 0.1 Hz-1 MHz - 최대 4개 입력 엘리먼트 |
| 상세사양 | WT3000 Precision Power Analyzer TYPE: 760304-04 <기본사양> ◎ Basic accuracy : 0.01% of reading ◎ Basic power accuracy 0.02% of reading ◎ Input Range Voltage : 15/30/60/100/150/300/600/1000 [V] Current (2A input element) Direct input : 5m/10m/20m/50m/100m/200m/500m/1/2/5 [A] External input : 50m/100m/200m/500m/1/2/5/10 [V] (opt.) (30A input element) Direct input : 500m/1/2/5/10/20/30 [A] External input : 50m/100m/200m/500m/1/2/5/10 [V] (opt.) ◎ Frequency power range : DC, and 0.1 Hz to 1 MHz ◎ Data update rate : 50 ms to 20sec ◎ Effective input range : 1% to 130% ◎ Communication function GP-IB, Serial port (RS232-C) or USB port (select one) (opt.) Ethernet interface (opt.), ◎ Display 8.4-inch TFT color LCD Numerical values, waveforms, trends, bar graphs, vectors - Simultaneous measurement with 2 units (8 power input elements) - Store function: 50 ms data storing interval Internal memory size : approximately : 30MB - Standard PC Card slot [옵션 및 기타사양] ◎ Motor evaluation function (/MTR) Calculation of motor and total efficiency Voltage, current, and power measurement with torque and speed input ◎ Advanced Calculation Function (/G6) harmonic measurement, IEC compliant harmonic measurement (requires harmonic measurement software), FFT calculation, waveform calculation functions, waveform Sampling Data Saving Function ◎ Delta Calculation (/DT) Checking phase voltage when you measure line to line voltage ◎ Added Frequency Measurement (/FQ) Checking the frequencies of all inputs ◎ D/A Output (/DA) Outputting measurement values as analog signals to other devices 20 parameters (each channel can be set separately) - Built-in printer(/B5) - VGA output (/V1) - USB Port(PC)(/C12) ◎ USB Port for Peripherals (/C5 Option) ◎ Cycle-by-cycle function (/CC) The function takes measurements of parameters such as voltage, current, and active power for each cycle, then lists the data on screen in a time series. - Flicker measurement (/FL) This function enables measurement of voltage fluctuations/flicker in compliance with EN61000-3-3 and IEC61000-3-11. Accessories function for above - Current Sensor Unit, 60Apk (CT60) 4 set - Conditional Box(4Ch) (CT60) 1 set [일반사양] 전원 100 - 240VAC 외관 426(W)×177(H)×459(D)mm 무게 App. 15kg (main unit with 4 input elements and options installed) |
| 표준액세서리 | Power cable, Power fuse, Current input protective cover, User manual, Safety terminal adapter, Rubber feet |